### MODELLING OF REGIONAL CHANGES OF RIVERINE NITROGEN FLOW IN THE SVARTA RIVER 1885-1995

L. Andersson (1), B. Arheimer (2)

(1) Department of Water and Environmental Studies, Linköping University

S-581 83 Linköping, Sweden

(2) Swedish Meteorological and Hydrological Institute, S-601 76 Norrköping

Sweden

- ABSTRACT

The aims of the study were to, in a 110 years perspective (1885-1995), for the lower Svartå basin:

- Estimate historical variation of factors of importance for net and gross load of riverine nitrogen

- Estimate historical variations of riverine nitrogen load, as a consequence of changes of these factors, and quantify the importance of the various environmental factors

- Estimate the effect of climatological variability on nitrogen loads

The HBV-N model was used to evaluate changes in nitrogen retention and transport caused by land drainage, lowering of lakes, building of dams and climatic variability in a river basin in south central Sweden (1885-1994). Rather modest conversions of waterbodies did only change nitrogen transport with about 1%. In general, dams were situated in locations more favourable for retention, compared to the lowered lakes. The almost two hundred times larger increase of (mainly) tile-drained agricultural land had, according to simulations, increased the nitrogen transport by 18%, due to reduced retention. However, compared to human-induced alteration of the retention in the landscape, the choice of 10-years period of climatological data had the overriding effect on the calculated nitrogen transport. Weather-induced variations resulted in 13% difference in nitrogen retention between various ten-year periods studied. The average nitrogen transport varied more than 60% between different ten-year periods during the 110 years modelled.